LibreVerter

Actions:

- Each of us write and share high level answers for overall aims:

What do we want to build?

-> real world example of durability, repairability, and also open source design…

-> to be able to use older technology semiconductors (COTS parts, common footprints, many 2nd/3rd/4th/... part substitution).

-> is it possible to develop it to be a base for an expandable design?

~ ~ ~ how to make it actually repairable (part level or board level modularity or other permutations???)

~ ~ ~ how to develop the intuitive interconnection structure?

What do we want to achieve?

What is the value we want to add?

PV -> MPPT -> Battery <-> DAB <-> DClink <-> Inverter <-> Load/Grid

Arief:

- Flow the overall aims into functional requirements -> Max 2kW

-> Generally it will be a 1kW bidirectional AC/DC converter with 60V battery storage and On/Off grid application.

-> System is DC coupled, means PV power is used to charge the battery before then the power is boosted to 400V via isolated converter to feed the inverter DClink.

-> MPPT converter will use boost converter -> after quick search, many of it has around 45V Voc

-> Increase battery voltage to 60V to simplify design to only use boost.

-> Standard OCP, OVP, UVP protections.

-> Switching between Off grid and On grid must be made manually.

Note:

1. on the contrary, some micro inverter are now designed as a single stage isolated converter PV->grid. Better efficiency and supposedly lower BOM cost, but complex technology and also different design philosophy.

2. its also possible to use 4 switch buck-boost to allow for 48V battery-> it may be beneficial in the possibility of lower battery cost.

https://imperix.com/

https://www.taraztechnologies.com/products/power-electronics/

https://www.pemodule.com/plugy-system-power-board-modules